Project Explanations

Experiment : Verification of Nortan’s Theorem

## Objective

To verify Nortan’s Thorem by measuring full load current.

# Procedure (Steps)

- Measurement of Original Load Current (IL)

Objective: To find the actual current flowing through the load resistor in the complex circuit.

  - Procedure: A DC voltage source was set to 15V. Three 330ohm resistors and one 1K load resistor RL were connected in a bridge-T configuration. A DC Ammeter (set to mA range) was placed in series with the 1Kresistor.

  - Observation: The ammeter measured the current IL, which serves as our reference value to verify Norton's Theorem

# Determination of Norton Current (IN)

Objective: To find the short-circuit current of the network.

  - Procedure: The 1K load resistor was removed from the circuit. The output terminals (where the load was) were "short-circuited" by connecting a DC Ammeter directly across them.

  - Observation: The current recorded on the ammeter is the Norton Current IN. This represents the maximum current the circuit can provide when the load resistance is zero.

# Determination of Norton Resistance (RN)

Objective: To find the equivalent internal resistance of the network.

  - Procedure: The 15V source was replaced with a short circuit (its internal resistance). The load remained disconnected. An Ohmmeter was connected across the open load terminals.

  - Calculation: Theoretically, we verified this by calculating the parallel combination of the first two 330 ohm resistors and adding the third:

# Norton Equivalent Circuit Verification

Objective: To prove that a single current source and one parallel resistor produce the same IL as the original complex circuit.

  - Procedure: A CSOURCE (Current Source) was placed and set to the IN value found in step 2 (15.15mA). A single resistor representing RN(495 ohm) was connected in parallel with the current source. Finally, the original 1K ohm load was connected in parallel to this combination.

  - Observation: The ammeter in series with the 1k load showed a current of 5.02mA, which matched the IL from the original circuit, thereby verifying Norton's Theorem