# Project Explanations

Project 1

Objective:
The aim of this project is to design a parallel circuit and calculate total current and current through each resistor.

Components Used:

- 9V DC Battery

- Resistors: R1 = 10kΩ, R2 = 2kΩ, R3 = 1kΩ

- Connecting wires

Procedure:

- Open Proteus and create a new project.

- Select components using Pick Devices (P).

- Add resistors and a DC battery.

- Place components on workspace.

- Set resistor values (10kΩ, 2kΩ, 1kΩ).

- Set battery voltage to 9V.

- Connect all resistors in parallel.

- Connect the circuit to the battery.

- Run the simulation.

Calculations:
1 / R_total = (1/10k) + (1/2k) + (1/1k)
1 / R_total = 0.0001 + 0.0005 + 0.001 = 0.0016
R_total = 625Ω

I_total = V / R = 9 / 625 = 14.4 mA

I1 = 9 / 10k = 0.9 mA
I2 = 9 / 2k = 4.5 mA
I3 = 9 / 1k = 9 mA

Project 2

Objective:
The aim of this project is to design a parallel circuit with four resistors and calculate total resistance and current in each branch.

Components Used:

- 9V DC Battery

- Resistors: R1 = 100Ω, R2 = 3kΩ, R3 = 5kΩ, R4 = 10kΩ

- Connecting wires

Procedure:

- Open Proteus and create a new project.

- Select components using Pick Devices (P).

- Add resistors and a DC battery.

- Place components on workspace.

- Set resistor values (100Ω, 3kΩ, 5kΩ, 10kΩ).

- Set battery voltage to 9V.

- Connect all resistors in parallel.

- Connect the circuit to the battery.

- Run the simulation.

Calculations:
1 / R_total = (1/100) + (1/3000) + (1/5000) + (1/10000)
1 / R_total = 0.01 + 0.00033 + 0.0002 + 0.0001 = 0.01063
R_total ≈ 94Ω

I_total = V / R = 9 / 94 = 95.7 mA

I1 = 9 / 100 = 90 mA
I2 = 9 / 3k = 3 mA
I3 = 9 / 5k = 1.8 mA
I4 = 9 / 10k = 0.9 mA