# Project Explanations

Experiment 1: Verification of KCL (Kirchhoff’s Current Law)

Objective:
To verify Kirchhoff’s Current Law (KCL) by measuring currents and proving that:
I1 = I2 + I3

Theory:
Kirchhoff’s Current Law states that total current entering a junction is equal to the total current leaving it.
In this circuit, current I1 enters the node and divides into I2 and I3.
So, I1 = I2 + I3

Procedure:

- Open Proteus and create a new schematic.

- Place a DC power supply (RPS).

- Add resistors:

  - 330Ω (left side)

  - 220Ω (right side)

  - 1kΩ (vertical branch)

  - 1kΩ (right vertical)

- Connect the circuit to form a junction.

- Place ammeters:

  - A1 for I1

  - A2 for I2

  - A3 for I3

- Connect ground properly.

- Set voltage to 5V, run simulation and note I1, I2, I3.

- Repeat for 15V and 25V.

- Record all readings.

Calculations:
Branch resistances:
R2 = 1kΩ
R3 = 220 + 1k = 1220Ω

Parallel resistance:
1 / Req = (1/1000) + (1/1220)

Total resistance:
RT = 330 + Req

Total current:
I1 = V / RT

Using current division:
I2 = I1 × R3 / (R2 + R3)
I3 = I1 × R2 / (R2 + R3)

Result:
It is observed that:
I1 ≈ I2 + I3
Hence, KCL is verified.

Experiment 2: Verification of KVL (Kirchhoff’s Voltage Law)

Objective:
To verify Kirchhoff’s Voltage Law (KVL) by measuring voltages and proving that:
E1 = V1 + V2 + V3

Theory:
Kirchhoff’s Voltage Law states that the sum of voltages in a closed loop is equal to the source voltage.

Procedure:

- Open Proteus and create a new circuit.

- Place a DC power supply (RPS).

- Add resistors:

  - 330Ω

  - 220Ω

  - 1kΩ

- Connect resistors in a closed loop (series).

- Place voltmeters:

  - V1 across 330Ω

  - V2 across 1kΩ

  - V3 across 220Ω

- Connect ground properly.

- Set voltage to 5V, run simulation and note V1, V2, V3.

- Repeat for 10V and 15V.

- Record readings.

Calculations:
Total resistance:
RT = 330 + 1000 + 220 = 1550Ω

Current:
I = V / RT

Voltage drops:
V1 = I × 330
V2 = I × 1000
V3 = I × 220

Verification:
E1 = V1 + V2 + V3

Result:
The sum of all voltage drops is equal to the source voltage.
Hence, KVL is verified.