# Project Explanations

### 1. Building the Non-Inverting Amplifier (Procedure)

- Step 1: Place the Op-Amp IC on the breadboard and connect the V+ (positive supply) and V− (negative supply) pins to the appropriate power rails.

- Step 2: Connect the non-inverting (+) terminal directly to the input signal source.

- Step 3: Connect a 1 kΩ resistor (R1) between the inverting (−) terminal and ground.

- Step 4: Connect a 2 kΩ resistor (R2) between the output terminal and the inverting (−) terminal (feedback path).

- Step 5: Ensure all ground connections (signal source, power supply, and circuit ground) are common.

### 2. Building the Inverting Amplifier (Procedure)

- Step 1: Place the Op-Amp on the breadboard and connect the power supply pins (V+ and V−) properly.

- Step 2: Connect the non-inverting (+) terminal directly to ground.

- Step 3: Connect the input signal to one end of a 1 kΩ resistor (R1), and connect the other end of R1 to the inverting (−) terminal.

- Step 4: Connect a 3 kΩ resistor (R2) between the output terminal and the inverting (−) terminal (feedback path).

- Step 5: Verify that all grounds are properly connected and there are no loose connections.